呼吸道處理設備建議

精  神

如美國麻醉醫師學會建議，特定呼吸道處置設備宜整合並易於攜帶。

The consultants and ASA members strongly agree that at least one portable storage unit that contains specialized equipment for difficult airway management should be readily available. Practice Guidelines for Management of the Difficult Airway. An Updated Report by the American Society of Anesthesiologists Task Force on Management of the Difficult Airway. Anesthesiology. 2013;118:251-70.

目  標

就本院現況，考量成本並且務實地將呼吸道處置設備分為基本及專家兩等級，並效法現有急救車的作法，統一擺放位置，以利不同單位間的互相支援。

建議設備

此處為基本建議(minimal requirement)，各單位可依實際需求增添或調整，務必以患者安全為優先。

基本級

面罩與甦醒器(BVM)，甚少處理呼吸道的單位可選用口袋面罩(pocket mask)

傳統式喉鏡(rigid laryngoscope with appropriate-size blades)

硬式(Yankauer)及軟式抽吸管(若牆上無內建式的抽吸設備，必須有攜帶型抽吸器)

氣管內管(適合該單位患者使用的尺寸)

聲門上呼吸道(supraglottic airways)(適合該單位患者使用的尺寸)

氣管插管前用藥(RSI medications)

血氧飽和度儀(pulse oximeter)

吐氣二氧化碳偵測器(exhaled CO2 detector)

專家級

設置於常需呼吸道處置的單位，如麻醉科、加護病房、急診等。

面罩與甦醒器(BVM)

傳統式喉鏡(rigid laryngoscope with appropriate-size blades)

影像式喉鏡(video laryngoscope with appropriate-size blades)

氣道通條(airway stylets)

氣道內視鏡(fiberoptic scope)

環甲膜切開(cricothyrotomy)設備

硬式(Yankauer)及軟式抽吸管(若牆上無內建式的抽吸設備，必須有攜帶型抽吸器)

氣管內管(適合該單位患者使用的尺寸)

聲門上呼吸道(supraglottic airways)(e.g., Laryngeal mask airway; LMA)

管插管前用藥(RSI medications)

血氧飽和度儀(pulse oximeter)

吐氣二氧化碳偵測器(exhaled Capnometer)